AIMS Environmental Science, 12(5): 835–855
DOI: 10.3934/environsci.2025037
Received: 14 December 2024
Revised: 29 July 2025
Accepted: 05 September 2025
Published: 28 September 2025

https://www.aimspress.com/journal/environmental

Research article

Machine learning-based LULC change detection and environmental implications in Bankura, West Bengal, India

Debabrata Nandi1, Rakesh Ranjan Thakur2,*, Bojan Ðurin3,*, Mayank Pandey4, Upaka Rathnayake5, Dillip Kumar Bera2 and Roshan Beuria6

- Department of Remote Sensing & GIS, Maharaja Sriram Chandra Bhanja Deo University, Baripada, Odisha-757003, India; debabrata.gis@gmail.com

- Centre of Remote Sensing and Disaster Management, School of Civil Engineering, Kalinga Institute of Industrial Technology, Deemed to be University, Bhubaneswar, 751024, Odisha, India; rakeshgeo@hotmail.com (R.R.T), dberafce@kiit.ac.in (D.K.B.)

- Department of Civil Engineering, University North-42000 Varazdin, Croatia; bojan.durin@unin.hr

- Technology Enabling Center, Guru Ghasidas University, Bilaspur, 495009, Chhattisgarh, India; makepandey1995@gmail.com

- Department of Civil Engineering and Construction, Faculty of Engineering and Design, Atlantic Technological University, F91 YW50 Sligo, Ireland; upaka.rathnayake@atu.ie

- Centre for Environment and Climate, Siksha ‘O’ Anusandhan Deemed to be University, Bhubaneswar, Odisha, India-751030; roshanbeuria922@gmail.com

*	Correspondence: Email: rakeshgeo@hotmail.com, bojan.durin@unin.hr.

Table S1. Distribution aerial coverage in km2 in the year of 1990.

|  | AG | DF | OF | BL | WB | BU | SS | Area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ML | 4441.1 | 676.3 | 434.5 | 208.1 | 303 | 803 | 93 | 6959 |
| RF | 4451.1 | 686.2 | 445.3 | 212.2 | 330.2 | 766 | 68 | 6959 |
| SVM | 4446.2 | 680.3 | 437.3 | 190.8 | 309 | 800.3 | 95.1 | 6959 |
| Average | 4446.1 | 680.9 | 439.0 | 203.7 | 314.0 | 789.7 | 85.3 |  |
| SD (±) | 5.0 | 5.0 | 5.6 | 11.4 | 14.3 | 20.6 | 15.1 |  |
| CV | 0.1 | 0.7 | 1.3 | 5.6 | 4.6 | 2.6 | 17.7 |  |

Note: Agriculture crop and fallow land (AG), Dense Forest (DF), Open Forest (OF), Barren land (BL), Water body (WB), Built-up area (BU) and Sandy soil (SS), Standard deviation (SD), and Coefficient of variance (CV).

Table S2. Distribution aerial coverage in km2 in the year 2000.

|  | AG | DF | OF | BL | WB | BU | SS | Area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ML | 4320.1 | 654.2 | 456.2 | 221.2 | 302.2 | 910.1 | 95 | 6959 |
| RF | 4420.1 | 664.2 | 466.2 | 226.3 | 328.3 | 781.9 | 72 | 6959 |
| SVM | 4336.2 | 660.2 | 460.5 | 198.7 | 304.3 | 903.1 | 96 | 6959 |
| Average | 4358.8 | 659.5 | 460.9 | 215.4 | 311.6 | 865.03 | 87.6 |  |
| SD (±) | 53.7 | 5.0 | 5.0 | 14.7 | 14.5 | 72.1 | 13.6 |  |
| CV | 1.2 | 0.8 | 1.1 | 6.8 | 4.7 | 8.3 | 15.5 |  |

Note: Agriculture crop and fallow land (AG), Dense Forest (DF), Open Forest (OF), Barren land (BL), Water body (WB), Built-up area (BU) and Sandy soil (SS), Standard deviation (SD), and Coefficient of variance (CV).

Table S3. Distribution aerial coverage in km2 in the year 2010.

|  | AG | DF | OF | BL | WB | BU | SS | Area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ML | 4290.1 | 638.1 | 470.2 | 224.4 | 301.5 | 938.4 | 96.3 | 6959 |
| RF | 4410.1 | 661.2 | 468.1 | 228.2 | 326.1 | 788.3 | 77 | 6959 |
| SVM | 4300.1 | 640.1 | 481.2 | 201.4 | 302.5 | 936.4 | 97.3 | 6959 |
| Average | 4333.43 | 646.467 | 473.167 | 218 | 310.033 | 887.7 | 90.2 |  |
| SD (±) | 66.6 | 12.8 | 7.0 | 14.5 | 13.9 | 86.1 | 11.4 |  |
| CV | 1.5 | 2.0 | 1.5 | 6.7 | 4.5 | 9.7 | 12.7 |  |

Note: Agriculture crop and fallow land (AG), Dense Forest (DF), Open Forest (OF), Barren land (BL), Water body (WB), Built-up area (BU) and Sandy soil (SS), Standard deviation (SD), and Coefficient of variance (CV).

Table S4. Distribution Aerial coverage in km2 in the year of 2024.

|  | AG | DF | OF | BL | WB | BU | SS | Area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ML | 4286.6 | 636.1 | 472.2 | 228.7 | 301.2 | 938.1 | 96.1 | 6959 |
| RF | 4408.1 | 660.2 | 474.1 | 230.2 | 301.2 | 797.2 | 88 | 6959 |
| SVM | 4266.6 | 618.3 | 502.6 | 226.8 | 298.1 | 948.4 | 98.2 | 6959 |
| Average | 4320.43 | 638.2 | 482.967 | 228.567 | 300.167 | 894.567 | 94.1 |  |
| SD (±) | 76.6 | 21.0 | 17.0 | 1.7 | 1.8 | 84.5 | 5.4 |  |
| CV | 1.8 | 3.3 | 3.5 | 0.7 | 0.6 | 9.4 | 5.7 |  |

Note: Agriculture crop and fallow land (AG), Dense Forest (DF), Open Forest (OF), Barren land (BL), Water body (WB), Built-up area (BU) and Sandy soil (SS), Standard deviation (SD), and Coefficient of variance (CV).

© 2025 the Author(s), licensee AIMS Press. This is an open access article distributed under the terms of the Creative Commons Attribution License (https://creativecommons.org/licenses/by/4.0)